Product Inspection Records

| Model | WT-A11988-Q06 |  |  |  | Item |  | Cavity Band Pass Filter |  |  |  |  | Quantity |  | 3pcs |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Appearance | Major Parameter |  |  |  |  |  |  |  |  | Other Parameter |  |  |  |  |
|  | Pass Band 1415 ~ 1425MHz F0=1420MHz |  |  |  |  | Stop Band |  |  |  | Connectors |  |  | Surface Finish |  |
| Reference value S/N | Insertion Loss | Ripple |  | Return Loss |  | - DC ~ 1370MHz |  |  | 1470 ~ 4500MHz | N-Female |  |  | Painted Black |  |
|  | ≤1.4dB | ≤0.3dB |  | ≥20dB |  | ≥75dB |  |  | ≥75dB |  |  |  |  |  |
| 1 | 1.13 | 0.06 |  | 25.1 |  | 80 |  |  | 78 | N-Female |  |  | Painted Black |  |
| 2 | 1.16 | 0.05 |  | 24.3 |  | 82 |  |  | 78 |  |  |  |  |  |
| 3 | 1.16 | 0.08 |  | 26.0 |  | 80 |  |  | 78 |  |  |  |  |  |
| Verdict: Inspection way：Full inspection Data recording mode：Full record |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Test Equipment：N5227B |  |  | Date：2026-03-06 |  |  |  |  | Tester：Liqiong Yong |  |  | Check： |  |  |  |