# SETI "Horn of Plenty" Flare Angle Calculations

Assumed typical dimensions:
Horn length L = 1.22 m
Aperture width A = 0.91 m
WR-650 throat width a = 0.165 m

## Nominal Flare Angle

Half-angle formula:

α = arctan((A − a) / (2L))

Substituting values:

α = arctan((0.91 − 0.165) / (2 × 1.22))

α = arctan(0.745 / 2.44)

α ≈ 16.97°

Included flare angle = 2α ≈ 33.94°

## With Additional 1 m Extension

New horn length L₂ = 2.22 m

α₂ = arctan((0.91 − 0.165) / (2 × 2.22))

α₂ = arctan(0.745 / 4.44)

α₂ ≈ 9.53°

Included flare angle = 2α₂ ≈ 19.06°

## Practical Tolerance

For amateur hydrogen-line horns, a practical build tolerance is approximately ±10–15% in flare angle before gain ripple and impedance mismatch become noticeable.

Therefore:

Original horn acceptable half-angle range: about 15° to 19°

Original horn acceptable included angle range: about 30° to 38°

Extended horn acceptable half-angle range: about 8.5° to 10.5°

Extended horn acceptable included angle range: about 17° to 21°

## Notes

These tolerances are practical engineering limits for hydrogen-line observing at 1420 MHz. Larger deviations mainly increase sidelobes and standing-wave ripple rather than completely preventing reception.