JS Pico Muon Examiner (3) 2026-Jul-23

A program to open and examine files generated by the UKRAA PicoMuon detector

- Although this is at a more advanced stage of development – it’s still best to think of it as a “Beta” version, especially as there are substantial recent function enhancements and a couple of irritating bugs.

- This version introduces two significant additional features:

  - A means of graphically selecting a “sub-range” of data in a loaded file for each slot. To do this click on the Filename text.

  - A means of generating synthetic data for this program

See later sections about each of these.

- Currently this program only works on files written to the SD card. (Future versions will be able to import files from an associated Raspberry Pi)

- This program only works on Windows machines

- You will need to have a reasonably up-to-date version of Java installed (It’s Ok with java version "11.0.15.1" 2022-04-22)

- To run it, double click on the “RunMuon.bat” file. You should see something like this:

- If you Drag an input file onto the window, after a while, when the data has been loaded, you should see something like this:

- You can select other plots using the “View” menu.

- There has been some attempt to check for poorly formatted data e.g. corrupt SD card. A log file is written into a subdirectory “Logs” where the file came from – have a look at that.

- Other known issues are:

  - The Information panel still needs work to make it look more even. If some information is hidden, resize the window.

  - The bottom axis is “double rendered” sometimes. Clicking on the window edge or resizing it fixes this.

  - Also editing this axis’ appearance isn’t working properly (see “Changing the display”)

  - Also,  (see “Changing the display”), pop-up graph editing windows are also inclined to appear in the wrong place, especially screen top left. And some features don’t work yet.

- Please note that the program will expire (not work) after a certain date. You should get warnings about this well ahead of that time.

“Slots”

“Slots” allow comparison of histograms of different files (and different parts of the same file – see later):

- You can add further files (or the same files again), just by dragging them into the program. They are registered in different slots and you will see the list of slots increasing as you do this.

- You can select which slot’s data is shown in the Information panel by selecting the appropriate button in the “Info” column. If one of the top four “View” options is selected, then the histogram plot will show the data for the selected slot.

- The View menu has three further options to allow comparison of the measured histograms of files in various slots. Which slots are included is controlled by the buttons in the “Hist” column.

- When comparing data from different slots and if “Normalise histograms” is not selected, you may well see large differences in displayed amplitudes. This is because files can have very different total event counts

New feature: Selecting sub-ranges:

If you have one or more files loaded, you can click on the filename and a new “Time series” window pops up looking a bit like this (for synthetic data with an abrupt change in rates):

You can change this view in several ways:

Via View  Plot Data you can select the data to be shown - Top detector, Coincidence (i.e. Muons) and Bottom detector rates.

Via  View  Binning you can select the size of the “bins” into which the data are collected. The higher the range the less the data fluctuates. Above a certain rate the graph is “squared up”.

Via  View  Show sub-range you can switch on and off the sub-range view.

You can change the sub-range by dragging the relevant triangle at the bottom. As soon as the mouse is release, the main view is of the sub-range data for the relevant slot. Many of the data fields in the main window turn yellow to denote this and show the sub-range statistics. You can then toggle between “full” and “sub-range” view with the relevant “Use sub-range” button.

This mechanism enables you to compare statistics for different parts of the same file. Just load the file multiple times – each time occupying a new slot. The sub-range for each slot can then be selected and using View->Compare xxx in the main window. Try this with the “PicoMuonTest.CSV” file included in this distribution.

Making synthetic files

Selecting View  Test file generator brings up anther window for making synthetic data files such as the “PicoMuonTest.CSV” one mentioned above. You fill in each Interval row in turn with the relevant information. (You need to press enter each time after entering a value. When rows are completed the Est totals row information becomes populated and the Make events button activates. Press this and a series of events will be generated. From File  Save you can save the file for loading into the main program.

Changing the graph displays

If you right-click on the graph, a menu should pop up allowing you to change the display parameters of the part of the graph that was selected. But this is still rather patchy and some windows pop-up in inconvenient places (e.g. top left of the screen.)

Feedback

I’d be very grateful if you would let me know of issues you encounter. Contact me at john@jegsav.com to do so. It would be helpful if you would copy and paste the end of the output in the associated Command window, especially error messages. Attach input files too, if they are not too large (8MB typically for email).

Good luck John Savage